# Gooee HVAC — Developer Guide (README_DEV.md)

## Overview

Gooee HVAC is a unified engineering platform combining: - Heat-loss modelling (U-values, constructions, glazing, infiltration) - Hydronic network simulation (pipes, circuits, emitters) - Balancing & pump sizing - Commissioning reports - Full project save/load in a single JSON format - Integrated GUI built in PySide6 (Qt)

This document provides developers with: - Project architecture - File structure - Coding conventions - Serialization structure - Extensibility guidance

## Directory Structure

HVAC/
├── core/
│   ├── heatloss/
│   │   ├── heatloss_elements.py
│   │   ├── construction_presets.py
│   │   ├── window_presets.py
│   │   ├── heatloss_json.py
│   │   ├── infiltration_model.py
│   │   └── building_wizard_cli.py
│   │
│   ├── hydronics/
│   │   ├── hydronic_network.py
│   │   ├── emitter_model.py
│   │   ├── friction_models.py
│   │   ├── balancing_engine.py
│   │   ├── pump_sizing.py
│   │   ├── commissioning_report.py
│   │   └── network_serialization.py
│   │
│   ├── project_io.py
│
├── gui/
│   ├── gui_gooee_main.py
│   ├── gui_room_editor.py
│   ├── gui_window_editor.py
│   ├── gui_construction_editor.py
│   ├── gui_uvalue_wizard.py
│   ├── gui_heatloss_wizard.py
│   ├── gui_hydronics_panel.py
│   ├── gui_balancing_panel.py
│   ├── gui_pump_panel.py
│   ├── gui_pipe_editor.py
│   ├── nodeview_building.py
│   └── nodeview_controller.py

## Architecture Summary

### 1. Heat-Loss Subsystem

- Room elements are composed of multiple Construction layers.

- Windows support multi-layer glazing systems.

- Heat-loss calculation: U-value × area × ΔT.

- Infiltration modelling: ACH-based, includes user-defined settings.

- Serialization: handled by heatloss_json.py.

### 2. Hydronic Subsystem

- Nodes: connection points in hydraulic topology.

- Pipes: length, diameter, roughness, loss coefficient.

- Emitters: radiators, UFH loops, etc.

- Circuits: ordered sets of pipes + emitters.

- Balancing: auto-calculates valve ΔP to equalize circuits.

- Pump sizing: determines design head, flow, and power.

- Commissioning output: text + CSV reports.

- Serialization: full network reconstruction via network_serialization.py.

### 3. Project Save/Load

- Stored as unified JSON with keys:

project = {
  "metadata": {},
  "building": { ... },
  "hydronics": { ... }
}

- Used by GUI (File → Save / Open).

### 4. GUI Architecture

- Built using PySide6 (Qt).

- Main window: gui_gooee_main.py.

- Uses QTabWidget for toolkit integration.

- Editors are embedded widgets with signals returned to the main controller.

- NodeView handles tree selection → editor updates.

## Serialization Format Overview

### Heat-Loss Structure

{
  "levels": [
    {
      "name": "Ground Floor",
      "rooms": [
        {
          "name": "Living Room",
          "elements": [...],
          "windows": [...]
        }
      ]
    }
  ]
}

### Hydronic Network Structure

{
  "nodes": { id: {"label": "..."}, ... },
  "pipes": { id: {...}, ... },
  "emitters": { id: {...}, ... },
  "circuits": { id: {"pipe_ids": [...], "emitter_ids": [...]}}
}

## Coding Standards

- All modules use PEP8 (within reason for engineering math).

- Classes use dataclasses where suitable.

- GUI uses clear MVC pattern:

  - Model: core/heatloss, core/hydronics

  - View: editors, tables

  - Controller: nodeview_controller.py

- Serialization is always bi-directional:

  - *_to_dict()

  - *_from_dict()

## Extensibility Guidance

### Adding New Heat-Loss Types

- Create new class extending base element.

- Add to heatloss_elements.py.

- Extend GUI editor or presets as required.

- Update heatloss_json.py to serialize new type.

### Adding New Emitters / Hydronic Components

- Define new Emitter class.

- Add to emitter_model.py.

- Update emitter_to_dict() and emitter_from_dict().

- Add GUI path if needed.

### Adding New Reports

- Add function to commissioning_report.py.

- Optional: attach to a new GUI tab.

### Adding New Project Sections

- Modify project_io.py to include new subsystem.

- Update main window load/save handlers.

## Developer Workflow

- Create or open project.

- Build heat-loss structure.

- Build hydronic circuits.

- Balance circuits and choose pump.

- Generate commissioning report.

- Save project for later.

## Future Roadmap

- DXF export for layouts.

- NodeView2 graphical hydronics diagram.

- Air-side modules (fans, ducts, AHU).

- Renewable systems (heat pumps, solar thermal).

- Full PDF export (reportlab).

- Theme engine (dark/light modes).

- Plugin architecture.

## Developer Contact Notes

This document is intended for internal development and contributors. An operator-facing guide should be written separately (README_USER.md).

End of file.